Command

Guidance

When the amount of information on the CLI interface is too much and inconvenient for user operation, or when the user does not need previous information, you can

use this command to clear the current screen information.

Usage Example

# Clear Screen Information

Switch#cl

s Switch#

# 2.1.5 configure

Command Function

The configure command can be used to enter the global configuration view from the privileged user view.

Command Form

configure

Parameter

Description

None

Default Value

None

Command View

Privileged User View

Command

Guidance

In the privileged user view, users can only view the complete operational status and statistics of the device, and can manage files and systems;

system management;

In the global configuration view, users can configure global parameters for the device in addition to the permissions in the privileged user view.

configuration.

Usage Example

# Enter the global configuration view from the privileged user view.

Switch#configure

Switch Command Line Manual 8

Chapter 2 Basic Commands

Parameter

Description

Parameter Description Values

level-value specifies the integer form of the command level, with a range of 0~15.

COMMAND specifies the command string to be set.

Default Value

None

Command View

Switch Command Line Manual 9

Chapter 2 Basic Commands

Switch(config)#

# 2.1.6 command-privilege level view

Command Function

The command 'command-privilege level view' can be used to set the command level within a specified view.

The command 'no command-privilege level view' can be used to restore the command to its default level.

Command Format

command-privilege level level-value view { configure | bgp | bgp-af-ipv4 | bgp-af-ipv4-

mcast | bgp-af-ipv4-vpn | bgp-af-ipv4-vpn-mcast | bgp-af-ipv6 | bgp-af-ipv6-vpn |

bgp-af-vpnv4 | bgp-af-vpnv6 | bgp-af-l2vpn | bgp-af-ipv4-label | bgp-af-vpls | aaa |

cpu-defend-policy | dhcp-pool | dhcpv6-pool | filter-lay2 | filter-ipv4 | filter-ipv6 |

filter-hybrid | g8032 | igmpsnoop-mvlan | igmp | igmp-vpn | isis | mld | mldsnoop-

mvlan | mlink | ip-vpn | ntp | ospf | ospfv3 | pim | pim6 | policy-route | isolate-group |

ripng | rip | rlink | route-policy | stp | time-range | schedule-profile | view | enable | if-

fe | if-ge | if-10ge | vlanif | eth-trunk | line | view | vlan } [ .COMMAND ]

no command-privilege level level-value view { configure | bgp | bgp-af-ipv4 | bgp-af-

ipv4-mcast | bgp-af-ipv4-vpn | bgp-af-ipv4-vpn-mcast | bgp-af-ipv6 | bgp-af-ipv6-vpn

| bgp-af-vpnv4 | bgp-af-vpnv6 | bgp-af-l2vpn | bgp-af-ipv4-label | bgp-af-vpls | aaa |

cpu-defend-policy | dhcp-pool | dhcpv6-pool | filter-lay2 | filter-ipv4 | filter-ipv6 |

filter-hybrid | g8032 | igmpsnoop-mvlan | igmp | igmp-vpn | isis | mld | mldsnoop-

mvlan | mlink | ip-vpn | ntp | ospf | ospfv3 | pim | pim6 | policy-route | isolate-group |

ripng | rip | rlink | route-policy | stp | time-range | schedule-profile | view | enable | if-

fe | if-ge | if-10ge | vlanif | eth-trunk | line | view | vlan } [ .COMMAND ]

Command

Guidance

The system manages commands hierarchically, with each command in each view having a specified level. Device administrators can base

on user needs to reset the command levels, allowing lower-level users to use some higher-level commands,

or to raise the command levels to enhance device security.

It is recommended that users do not modify the default levels of commands without professional guidance to avoid operational and maintenance issues,

and even pose security risks to the device.

Usage Example

# Set the level of the cd command to 5.

Switch(config)#command-privilege level 5 view configure

cd Switch(config)#

# 2.1.7 debug cli

Command Function

The debug cli command can be used to enable CLI debugging.

The no debug cli command can be used to disable CLI debugging.

Command Format

debug cli { cmd | pty | auth | exec-config | show-config | sync | telnetd }

no debug cli { cmd | pty | auth | exec-config | show-config | sync | telnetd | all }

Parameter

Description

None

Default Value

None

Command View

Privileged User View

Command

Guidance

None

Switch Command Line Manual 10